INVESTIGATION OF ATTITUDE CONTROL SYSTEM OF A PITCH BIAS MOMENTUM EARTH POINTING SPACECRAFT

# INTRODUCTION

In this piece of work an arbitrary satellite model is developed and its attitude control system is studied. The system is discussed from different points. The satellite model possesses a momentum wheel on pitch axes, and thruster couples about roll and yaw axes. The orbit is assumed to be a circular orbit in order to simplify the equations.

# DYNAMIC EQUATIONS AND SYSTEM MODEL

## REFERENCE FRAMES

Zo

Xo

Yo

Y1

X1

Z1

n

Z2

Y2

X2

Figure 2-1: Reference frames

Orbit

To start with the modelling of dynamic equations we first choose three co-ordinate systems. The first co-ordinate system is the absolute (inertial) frame which is indicated by “Xo Yo Zo” (which will be referred to as “frame [0]” in the proceeding text) in figure 2-1. The origin of this inertial system is assumed to be at the centre of the earth and is assumed to be fixed. (i.e. it does not move with respect to earth). The second reference frame is denoted by “X1 Y1 Z1” (which will be referred to as “frame [1]” in the proceeding text) in figure 2-1. This system is an intermediate system between “X2 Y2 Z2” system (which will be referred to as “frame [2] in the proceeding text) which is the satellite body frame. The “X1 Y1 Z1” rotates about Y1 with the orbital rate ν. Due to the fact that the orbit is circular, this rate is constant throughout one orbital period. Frame [2] is fixed to the body of the satellite with its origin being at the centre of mass of the satellite.

## SATELLITE MODEL

The satellite is assumed to have a cubical shape (i.e. all sides have equal length) with a homogeneous mass distribution. These features simplify the equations further as the inertia matrix is now a diagonal matrix. (i.e. non-diagonal terms are zero) The roll axis is the X2 axis, pitch axis is Y2 axis and the yaw axis is the Z2 axis in figure 2-1.Any rotation about roll axis will be denoted by f, any rotation about pitch axis will be denoted by q and any rotation about yaw axis will be denoted by y. The momentum wheel is located at the origin (i.e. at the mass centre) and aligned parallel to the pitch axes thus the rotations about pitch axis may be controlled by this wheel. The two couples of thrusters are located on the outer facets of the satellite to give torque about roll and yaw axes. Thus the rotations about roll and yaw axes are controlled by these thrusters. (see figure 2-2)

Figure 2-2: Satellite model

Momentum wheel

Yaw thrusters

Roll thrusters

X2 (roll)

Y2 (pitch)

Z2 (yaw)

## EQUATIONS OF MOTION

Unless otherwise stated; a bold letter will denote the related vector , an underlined bold letter will denote the related dyadic, an underlined letter will denote the related matrix (or a column) representation. Recall Euler’s equation;

t2 = D0 H2	[2-1]

t2 : sum of all torques acting about origin of frame [2]

D0 : derivation with respect to time in frame [0]

H2 : sum of all angular momentums acting about origin of frame [2]

equation [2-1] may be re written as;

t2 = D0 H2 = D2 H2 + w2 ´ H2	[2-2]

w2 : angular velocity of frame [2] with respect to frame [0]

D2 : derivation with respect to time in frame [2]

H2 = Hw + Hsc		[2-3]

Hw : Angular momentum of wheel

Hsc : Angular momentum of spacecraft

Hw = Iw ww		[2-4]

Iw : inertia dyadic of wheel

ww : absolute angular velocity of wheel

Hsc = Isc wsc		[2-5]

Isc : inertia dyadic of satellite

wsc : absolute angular velocity of satellite

ww = ww/sc + wsc/e	[2-6]

ww/sc : angular velocity of wheel with respect to spacecraft

wsc/e : angular velocity of spacecraft with respect to earth (absolute frame)

Substituting equations [2-4] through [2-6] in equation [2-3] yields;

H2 = Iw (ww/sc + wsc/e) + Isc wsc	[2-7]

then substituting equation [2-7] in equation [2-2] we have;

t2 = D2 {Iw (ww/sc + wsc/e) + Isc wsc}+ w2 ´ {Iw (ww/sc + wsc/e) + Isc wsc}	[2-8]

Equation [2-8] is the dynamic equation of motion for the satellite. This vector representation may well be expressed in any frame. Nevertheless the inertia dyadic appears to be constant (i.e. does not change by time) in body frame thus giving rise to a constant inertia matrix. So body frame is the best frame to express this vector equation. In body frame we have;

t2 = D2 {Iw (ww/sc + wsc/e) + Isc wsc}+ w2 {Iw (ww/sc + wsc/e) + Isc wsc}	[2-8]

The terms in equation [2-8] when expressed explicitly appear as follows;

Iw=J1000J2000J1		[2-9.a]

wwsc=0Ω0			[2-9.b]

wsce=wsc=w2=wxwywz	[2-9.c]

Isc=Ix000Ix000Ix		[2-9.d]

w2=0−wzwywz0−wx−wywx0	[2-9.e]

Substituting equations [2-9] in equation [2-8] we have;

txtytz=J1+Ixwx+wywzJ1−J2−J2ΩwzJ2Ω+J2+IxwyJ1+Ixwz+wxwyJ2−J1+J2wxΩ	[2-10]

Equation [2-10] gives the full dynamics of the satellite. The torque components on the left hand side are the external torques applied about the centre of mass (origin of frame [2]) of satellite. Note the pitch dynamics is independent from the roll and yaw dynamics while roll and yaw dynamics are coupled. (i.e. affect each other)

## ANGULAR VELOCITIES AND EULER ANGLES

The control of satellite is managed via using euler angels thus we need a conversion from angular velocities to euler angles. The angular velocities arise due to motion of frame [1] with respect to frame [0] (a rotation about Y1 axis) and due to motion of frame [2] with respect to frame [1] (see figure 2-1). Thus we need to express the transformation from frame [0] to frame [2].

First consider transformation from frame [0] to frame [1].

Figure 2-3 : Transformation from frame [0] to frame [1]

u2(0)

[1]

[0]

n

In figure [2-3] the transformation from frame [0] to frame [1] is defined as “rotation about second unit vector of frame [0] (i.e. u2(0)) with an angle of n”. This rotation is defined as “second elementary rotation” because rotation takes place about second axis (i.e. Y axis) only! Thus the transformation matrix from frame [1] to frame [0] appears as;

R2(0,1)=cos(ν)0sin(ν)010−sin(ν)0cos(ν)	[2-11]

This matrix transforms any column in frame [1] to frame [0], i.e. [0] = R2(0,1) [1].

Next consider transformation from frame [1] to frame [2] in the same manner.

f

Figure 2-4 : Transformation from frame [1] to frame [2]

q

y

[1]

[a]

u1(1)

[b]

[2]

u2(a)

u3(b)

In figure [2-4], transformation from frame [1] to frame [2] is shown. The frames [a] and [b] are only intermediate frames used to help understanding the rotation. The transformation sequence is not unique. The sequence used here is; a rotation about first unit vector of frame [1] by an angle f (roll), then a rotation about second unit vector of frame [a] by an angle q (pitch), then finally a rotation about third axis of frame [b] by an angle y (yaw). These are also elementary rotations (first, second and third elementary rotation respectively) since the rotations take place about only one axis at a time. The related transformation matrices appear as follows;

R1(1,a)=1000cos(f)−sin(f)0sin(f)cos(f)	[2-12]

R2(a,b)=cos(q)0sin(q)010−sin(q)0cos(q)	[2-13]

R3(b,2)=cos(y)−sin(y)0sin(y)cos(y)0001	[2-14]

The matrices given in equations [2-12] through [2-14] transform any column in frame [2] to frame [1], i.e. [1] = R1(1,a) R2(a,b) R3(b,2) [2].

In the light of these explanations the angular velocity of frame [2] with respect to frame [0] (absolute frame) can be written as (in vector form);

w= {νu2(0)+fu1(1)+qu2(a)+yu3(b)	[2-15]

Note since the orbit was chosen to be a circular one, n varies linearly with time i.e. n = w0×t in which w0 is the orbital rate of the satellite. Thus νis constant i.e. ν= w0 . Now that equation [2-15] can be re written as;

w=w0u2(0)+fu1(1)+qu2(a)+yu3(b)	[2-16]

As explained earlier, we are always in need to express the equation in body frame (satellite fixed frame) thus to express [2-16] in body frame (frame [2]) we can write;

w = w0 u2(0/2) + f u1(1/2) + q u2(a/2) + yu3(b/2) 	[2-17]

ui(j/k) : representation of ui of frame [j] in frame [k]

There are two points worth considering at this stage:

- Recall figure [2-1] and note that Y1 of frame [1] and Y0 of frame [0] always coincide. Thus we have

u2(0/2) = u2(1/2)		[2-18]

This is a relief because in the transformation matrix for frame [0] to frame [1] (see equation [2-11]) we have cos (n) and sin (n) terms which are obviously dependent on time due to     n = w0 × t. Obviously this fact would have given rise to transformation matrices dependent on time! Due to [2-18] we can use matrices given in equations [2-12] through [2-14] (which are obviously are not explicit functions of time) instead of the matrix given in [2-11]. This point is of critical importance for control aspects which will be discussed later in the proceeding text.

- Now consider the term u3(b/2) in equation [2-17]. Note that last rotation in figure [2-4] takes place about the third axis of frame [b]. This means third axis remains unchanged stating that third unit vectors of frame [b] and frame [2] coincide, i.e.;

u3(b/2) = u3(2/2) 		[2-19]

By the help of [2-18] and [2-19], we can re write [2-17] as;

w = w0 u2(1/2) + f u1(1/2) + q u2(a/2) + yu3(2/2) 	[2-20]

The following transformations will help evaluating equation [2-20].

u2(1/2) = R(2,1) u2(1/1)

u2(1/1) = [0 1 0]T

R(2,1) = [R(1,2)]T = [R1(1,a) R2(a,b) R3(b,2)]T

R(2,1)=cos(q)cos(y)sin(f)sin(q)cos(y)+cos(f)sin(y)−cos(f)sin(q)cos(y)+sin(f)sin(y)−cos(q)sin(y)−sin(f)sin(q)sin(y)+cos(f)cos(y)cos(f)sin(q)sin(y)+sin(f)cos(y)sin(q)−sin(f)cos(q)cos(f)cos(q)u2(12)=sin(f)sin(q)cos(y)+cos(f)sin(y)−sin(f)sin(q)sin(y)+cos(f)cos(y)−sin(f)cos(q)	[2-21]

u1(1/2) = R(2,1) u1(1/1)

u1(1/1) = [1 0 0]T

u1(12)=cos(q)cos(y)−cos(q)sin(y)sin(q)	[2-22]

u2(a/2) = R(2,a) u2(a/a)

u2(a/a) = [0 1 0]T

R(2,a) = [R(a,2)]T = [R2(a,b) R3(b,2)]T

R(2,a)=cos(q)cos(y)sin(y)−sin(q)cos(y)−cos(q)sin(y)cos(y)sin(q)sin(y)sin(q)0cos(q)

u2(a2)=sin(y)cos(y)0	[2-23]

u3(2/2) = [0 0 1]T	[2-24]

Substituting equations [2-21] through [2-24] into [2-20]we have;

wxwywz=w0sin(f)sin(q)cos(y)+cos(f)sin(y)+fcos(q)cos(y)+qsin(y)w0−sin(f)sin(q)sin(y)+cos(f)cos(y)−fcos(q)sin(y)+qcos(y)−w0sin(f)cos(q)+fsin(q)+y  [2-25]

Equation [2-25] shows the relation between angular velocities and the euler angles. We also need the derivatives of angular velocities, i.e.;

wx=fcos(q)cos(y)+qsin(y)+f−w0sin(f)sin(y)+w0sin(q)cos(y)cos(f)+qw0sin(f)cos(q)cos(y)+y−w0sin(f)sin(q)sin(y)+w0cos(f)cos(y)+fq−sin(q)cos(y)fy−cos(q)sin(y)+qy(cos(y))[2-26]

wy=f−cos(q)sin(y)+qcos(y)+f−w0cos(f)sin(q)sin(y)−w0sin(f)cos(y)+q−w0sin(f)cos(q)sin(y)+y−w0sin(f)sin(q)cos(y)−w0cos(f)sin(y)+fqsin(q)sin(y)+fy−cos(q)cos(y)+qy−sin(y)[2-27]

wz=fsin(q)+y+f−w0cos(f)cos(q)+qw0sin(f)sin(q)+fqcos(q)	[2-28]

Now that equations [2-25] through [2-28] may be substituted in equation [2-10] to give the full dynamics of the satellite in terms of torque and euler angles.

## SIMPLIFIED EQUATIONS OF MOTION

Since the equations given by [2-25] through [2-28] are highly non linear and coupled; when we speak in terms of control; it is extremely difficult to suit a control algorithm by using these equations. For small deviations from stability, (i.e. if the values for deviations in roll, pitch and/or yaw are small) we can use the small angle assumption. This assumption simplifies the equations of motion such that we can apply linear control laws easily. Let;

f = f0 + df		[2-29]

q = q0 + dq		[2-30]

y = y0 + dy		[2-31]

where f0 , q0 , y0 denote the original values (bias values) for roll pitch and yaw angles respectively and df, dq, dy denote the deviations from equilibrium for roll, pitch and yaw respectively. For the time being assume the bias values are zero (0) and thus we have;

f = df		[2-32]

q = dq		[2-33]

y = dy		[2-34]

When we talk about “small angle assumption” we also say that;

- provided that angles are in radians;

- “sin” of a small angle is equal to itself, i.e.:sin(df) @ df, sin(dq) @ dq, sin(dy) @ dy;

- “cos” of a small angel is equal to “1”, i.e.:cos(df) @ 1, cos(dq) @ 1, cos(dy) @ 1;

- the product of two small angles may be ignored, i.e.: df.dq @ 0, df.dy @ 0, dq.dy @ 0.

Thus now equations [2-12] through [2-14] may be re written as the following;

R1(1,a)=10001−df0df1	[2-35]

R2(a,b)=10dq010−dq01	[2-36]

R3(b,2)=1−dy0dy10001	[2-37]

Then equations [2-21] through [2-23] become;

u2(12)=dy1−df		[2-38]

u1(12)=1−dydq		[2-39]

u2(a2)=dy10			[2-40]

Substituting equations [2-38] through [2-40] and equation [2-24] into equation [2-20] yields;

w=w0dy1−df+df1−dydq+dqdy10+dy001	[2-41]

ignoring products of small angles [2-41] can further be simplified to give;

wx=w0dy+df	[2-42]

wy=w0+dq		[2-43]

wz=-w0df+dy	[2-44]

and the derivatives become;

wx=w0dy+df	[2-45]

wy=dq		[2-46]

wz=-w0df+dy	[2-47]

Now substituting equations [2-42] through [2-47] into equation [2-10] we have;

tx=w0J1+Ixdy+J1+Ixdf+w02J2−J1df+w0J1−J2dy+J2w0Ωdf−J2Ωdyty=J2Ω+J2+Ixdqtz=-w0J1+Ixdf+J1+Ixdy+w02J2−J1dy+w0J2−J1dq+w0J2−J1df+w0J2Ωdy+J2Ωdf

tx=w02J2−J1+J2w0Ωdf+w0J1+Ix+w0J1−J2−J2Ωdy+J1+Ixdfty=J2Ω+J2+Ixdqtz=

[2-48]

Note in equations [2-48] w0 (orbital) rate is constant since we have assumed the orbit as a circular orbit. Furthermore all moments of inertia (J1, J2, Ix) are constant. Thus equation is linear in terms of deviations. Note pitch dynamics is not affected from roll and/or yaw dynamics, however roll and yaw dynamics are coupled. (i.e. affect each other).

# CONTROL OF SIMPLIFIED MODEL

To fit a suitable control algorithm for the system we will consider the set of equations [2-48], and we will also make use of modern control theory due to the fact that the system is multi-input multi-output (i.e. inputs are torque due to thrust, torque due to momentum wheel acceleration outputs are 3 euler angles).

Note in the equations [2-48] that the pitch dynamics is independent of roll and yaw dynamics and can be controlled via Ω(momentum wheel acceleration and thus the torque) or ty (torque about pitch axis). In the same manner roll and yaw dynamics can be controlled via tx and tz (torque about roll and yaw axes). Also note that roll-yaw dynamics is affected by wheel speed W which is driven by pitch dynamics. This means if there is no variation in pitch dynamics control input (i.e. wheel torque) then the coefficients in roll-yaw dynamics are constant. Thus the eigenvalues of roll-yaw dynamics are constant if there is no variation in wheel speed. However a control system for pitch dynamics will drive the pitch loop continuously causing the wheel speed change all the time. Thus the designed eigenvalues of roll-yaw dynamics will continuously change. Nevertheless various control systems for roll-yaw dynamics may still be designed such that the system will be stable at all times. This fact must be considered in design step.

## NUMERICAL VALUES

The numerical values corresponding to the specifications of the system are needed in order to go further with the simulations.

### SATELLITE SPECIFICATIONS

Let the satellite be a cube with one side 80 cm and mass 75 kg. Then the moments of inertia appear as follows;

a = 80 . 10-2 m

m = 75 kg

Ix = Iy = Iz = (1/12) m (a2 + a2) = 8 kg m2

(These figures are assumed)

### THE MOMENTUM WHEEL SPECIFICATIONS

For a realistic system the following momentum wheel specifications correspond to a real momentum wheel which is commercially available in the market by ITHACO systems.

m = 12 kg

r = 18 cm

J1 = (1/4) m r2 = 0.0972 kg m2

J2 = (1/2) m r2 = 0.1944 kg m2

Figure 3.1: Moments of inertia of momentum wheel

J2

J1

### THE SPEED OF THE WHEEL

Recall we require a locked rotation, i.e. an earth pointing satellite. To find the required wheel speed which will perform this task consider equation [2-7] which is the total angular momentum acting about origin of frame [2]. Express this equation in body frame;

hxhyhz=J1000J2000J1wxwy+Ωwz+Ix000Ix000Ixwxwywz		[3-1]

hx = J1 wx + Ix wx		[3-2]

hy = J2 (wy + W) + Ix wy	[3-3]

hz = J1 wz + Ix wz		[3-4]

The nominal wheel speed W may be found via equation [3-3]. Solving this equation for W yields;

Ω=hy−2w0Ix+J2J2	[3-5]

in which;

w0=2pT=2p2pa3m=ma3	rad s-1 		[3-6]

We want a momentum biased system, thus now assume the angular velocity of the wheel has been increased until 4000 rpm (418.880 rad s-1 ) and conserve this velocity. Then the total angular momentum of the satellite will be;

hw = w0 J2 = 418.880 * 0.1944 = 81.43 kg m2 s-1 	[3-7]

Since the total angular momentum will be conserved after the satellite has started its motion, the total angular momentum will always be 81.43 kg m2 s-1. Then by the help of [3-5];

Ω=81.43−2(1.06e−3)(8+0.1944)0.1944=418.789 rad s-1	[3-8]

The angular velocity of the wheel obtained by [3-8] is the nominal operating value and will deviate (+ or -) from this value due to disturbances.

## CONTROL ALGORITHMS

Consider again the equations [2-48]. Recall these are the simplified equations of motion and may be used for control process for small deviations from the equilibrium. Note roll and yaw dynamics is not dependent on pitch dynamics (except the changing wheel speed due to control in pitch) so the control process will be studied in two different parts (for pitch and for roll-yaw) then the results will be combined to study the overall system. The inputs of the system are assumed to be tx , tz and Ω .Note tx provides control for roll dynamics, tz provides control for yaw dynamics. ty is assumed to be zero (i.e. no torque about pitch axis) since there already exists a control via wheel (i.e. Ω).

### PITCH CONTROL

Since the pitch loop is simpler, the control on pitch dynamics will be studied first. Defining states as;

x1ºdq	[3-9]

x2ºdq	[3-10]

the input as;

uºΩ		[3-11]

and also remembering that ty º 0 we can re write pitch dynamics as;

x1=x2x2=−J2J2+Ixu	[3-12]

Equations [3-12] may be written in matrix form as follows;

x1x2=0100x1x2+0−J2/J2+Ixu	[3-13]

Substituting J2 = 0.1944 (given in section 3.1.2) and Ix = 8 (given in section 3.1.1) equation [3-13] becomes;

x1x2=0100x1x2+0−0.024u		[3-14]

Assuming all states are available (i.e. measured precisely) we can apply a state feed back control in which;

u = -K x 	[3-15]

K : gain matrix of which the elements are to be calculated

x : state vector

Since the input is single and the state vector is a [2 x 1] column matrix, the gain matrix must have the dimension [1 x 2] i.e. K = [k1 k2]. By the help of [3-15] we can re write equation [3-14] as follows;

x=A−BKx	[3-16]

A : system matrix

B : input matrix

thus;

x=010.024k10.024k2x	[3-17]

Let all the eigenvalues of this system be located at l=-0.5 (i.e. double eigenvalues). When the eigenvalue problem is solved we find the gain values as;

k1 = -10.4

k2 = -41.7

Since we defined the input of the system as a function of states we have;

u = -K x = -(-10.4 x1 - 41.7 x2) = 10.4 x1 + 41.7 x2 		[3-18]

as the input. The physical interpretation of [3-18] is that the input (i.e. the acceleration of the wheel) should be adjusted by the wheel driving computer according to equation [3-18]. (This is just a problem of measuring states since we assume the states are observable). Also note that for the states we have the following relation ship;

x1=x2	[3-19]

x2=0.024k1x1+0.024k2x2=0.024k1x1+k2x2=-0.024u	[3-20]

In what follows we will present the MATLAB simulation results for the designed pitch control system. Figure 3-2 shows the block diagram prepared in simulink. To start simulation an initial value of 0.5 radians is chosen as a deviation for pitch. Figure 3-3 shows the variation of dq and dqfor a period of 20 seconds. Note the system stabilizes itself (i.e. reaches equilibrium) in almost 10 seconds. Figure 3-4 is a plot of how the control input (i.e. wheel acceleration) varies in time.

Figure 3.2: Matlab simulation blocks for pitch control

Figure 3.3: Variation in theta and theta_dot

Figure 3.4: Variation in omega_dot

To decrease the settlement time we can push the eigenvalues of the system further to the left of imaginary axis. This time for example choose the eigenvalues to be l=-1.5 and re solve the eigenvalue problem for the equation [3-17]. The gain coefficients may found to be;

k1 = -93.75		[3-21]

k2 = -125		[3-22]

If these values are substituted in the system given in figure [3-2], the results given in figures [3-5] and [3-6] below may be obtained.

Figure 3.5: Variation in theta and theta_dot with new gain values

Figure 3-6: Variation in omega_dot

As can be seen from figure [3-5] and [3-6] the settlement time has been improved from about 10 seconds to 5 seconds; i.e. the same error in pitch can now be corrected in about five seconds.

The foregoing discussion tells that it is mathematically possible to improve the system further. However another point to be discussed should be whether or not it is possible to realise such a system physically. (i.e. availability of application of such gain coefficients)

### ROLL YAW CONTROL

Recall equations [2-48] and this time consider roll and yaw dynamics. Let’s define state in the following fashion;

x1ºdf		[3-23]

x2ºdy		[3-24]

x3ºx1=df		[3-25]

x4ºx2=dq		[3-26]

and let’s define the inputs in the following fashion;

u1ºtx		[3-28]

u2ºty		[3-29]

thus the roll and yaw dynamics given in equations [2-48] may be written as follows;

u1=w0J1+Ixx4+J1+Ixx3+w02J2−J1x1+w0J1−J2bitmedi...